Edger Sander– Instructions for use

USES- Sanding edges of floor areas

PRE-START CHECKS:

Extension lead is safely positioned.

Machine is switched off before connecting to mains power supply.

Nail down any loose boards and punch down nails.

Make sure any glue is removed from the floor.

Dust bag and spanner are with the machine and correctly fitted.

OPERATION:

Fit new disc to machine using spanner provided.

Tilt machine back on castors and switch on.

Lower machine onto the floor and move in a forward motion only.

Weight of the machine is sufficient for a score free finish.

DO NOT press down.

Empty dust bag frequently when 1/3 full and at completion of job.

STOPPING AND AFTER USE:

Tilt machine backwards at end of run.

Switch off and allow disc to stop turning before placing back down.

SAFETY PRECAUTIONS:

Unplug machine from mains power supply when changing discs.

EXTENSION CORD TO BE A MAXIMUM OF 6 METRES ONLY

Keep extension lead clear of work area.

DO NOT wear loose fitting clothing.

HAZARDS:

Be aware of not damaging extension lead when operating.

Empty dust bag frequently – sawdust is combustible and can be a fire hazard.

CAUTION – Some floor coverings may contain ASBESTOS, sanding can pose a health risk if inhaled.

SAFETY EQUIPMENT