Heat Gun/Paint Stripper – Instructions for use

OPERATION:

- Plug the gun into the power supply.

- Set heat setting to a low setting & test on a s small area holding gun 10cm from the surface.

- Heat the surface. Using a scraper to remove paint the heat setting can be increased or decreased as required.

- Be careful not to have the heat setting too high as this could damage the surface.

STOPPING AND AFTER USE:

- Switch the gun off and let cool down. Be careful where you place the gun while cooling down as the metal nozzle will be hot for some time.

- Unplug from power supply.

SAFETY PRECAUTIONS:

- Do not obstruct either the air intake or nozzle outlet, as this may cause excessive heat build-up resulting in damage to the tool.

- Do not direct the hot air blast at other people.

- Do not touch the metal nozzle as it becomes very hot during use and remains hot for up to 30 minutes after use.

- Do not poke anything down the nozzle.

- Do not look down the nozzle while the unit is working.

- Do not use as a hairdryer.

REQUIRED SAFETY EQUIPMENT: