Leaf Blower/Vac Petrol – Instructions for use

OPERATION:

- Place the unit on the ground so that it is secure.

- Make sure you have a firm footing: Hold the unit firmly with your right hand on the housing and press down.

- Hold the starter grip with your left hand.

- Pull the starter grip slowly until you feel it engage and then give it a brisk strong pull.

- Do not let the starter grip snap back. Guide it slowly back into the housing so that the starter rope can rewind properly.

Using the Blower

- Your blower is designed for one-handed operation.

- It must be carried by the control handle in the right hand.

- Always hold the power tool firmly in your hand.

- Walk slowly forwards as you work – observe the nozzle outlet at all times.

Using the Vacuum

- Your vacuum shredder is designed for two-handed operation.

- Hold and operate the unit with your right hand on the control handle and your left hand on the assist handle.

- Wear the catcher bag shoulder strap over your left shoulder, not across your chest. This enables you to quickly remove the power tool and catcher bag in an emergency.

STOPPING AND AFTER USE:

- Move the stop switch to 0 – the

- engine stops – the stop switch springs back to the on position.

SAFETY PRECAUTIONS:

- Watch out for small animals when using the blower on open ground, in yards and gardens.

- Do not walk backwards – risk of stumbling.

REQUIRED SAFETY EQUIPMENT: