Domestic Motor Mower – Instructions for use

PRE-START CHECKS:

- Secure handle position with locking knobs.

- Set cutting height.

- Fit catcher if required.

- Check the area for debris eg wire, stones etc.

- Check that all guards are in place and the catcher is properly attached.

STARTING PROCEDURE:

- Engage choke (cold start).

- Hold mower steady with one hand & engage safety/dead man lever.

- Grasp starter pull cord and pull evenly.

- When engine starts move throttle to run position.

- Disengage choke.

OPERATION:

- When catcher is full, turn off mower & empty.

- Do not leave engine running while unattended.

- If vibration occurs stop immediately.

STOPPING AND AFTER USE:

- Move throttle control to idle.

- Release safety lever.

- Remove catcher.

- Allow to cool down.

- Remove bulk of clippings from under carriage and catcher.

SAFETY PRECAUTIONS:

- Keep children away.

- Do not empty catcher while motor is running.

- Do not clean out grass with hands while engine is running.

- Keep hands and feet clear of mower.

REQUIRED SAFETY EQUIPMENT