Plate Compactor – Instructions for use

PRE-START CHECKs:

- Ensure there are no foreign objects -wire, steel etc.

- Use extreme caution & always be aware of ground subsidence with compaction vibration transfer.

OPERATION:

- Turn ignition switch to ON position.

- Engage choke (cold start).

- Move throttle lever to half throttle.

- Pull starter cord with short sharp pulls until engine fires or starts.

- Release choke as soon as engine fires up.

- Move throttle lever to the idle position.

- Allow engine to warm up.

- Hold compactor firmly with both hands.

- Move throttle lever to desired speed.

- Proceed to steer compactor around area to be compacted.

- At the end of the compaction line steer the compactor around as these units do not operate backwards.

STOPPING AND AFTER USE:

- Idle engine

- Turn ignition switch to OFF.

SAFETY PRECAUTIONS:

- Hold compactor with both hands.

- Make sure you have a firm footing when operating.

- If the site is level you shouldn’t have to push or pull the machine around, the vibration action is designed to make the machine go forward.

- Ensure the work area is clear of any obstacles, steep gradients or uneven ground.

REQUIRED SAFETY EQUIPMENT: