Hydraulic Post Hole Borer– Instructions for use

PRE-START CHECKS:

- Check auger size for machine and job.

- Check cutters on auger.

- Check fuel tank is full (unleaded 91)

STARTING PROCEDURE:

- Turn on engine switch.

- Engage the choke if starting from cold.

- Pull starter cord out until a slight resistance is felt, let starter recoil, and give a sharp pull till engine fires or starts.

- Disengage the choke.

- Allow engine to warm up.

- To fit auger: position auger into coupling on hydraulic drive motor.

- Push up collar and release to lock auger into position.

- Lift machine to check that auger is fully locked into position.

OPERATION:

- Position borer where hole is to be bored using self-drive hydraulic lever (some models only)

- Activate hydraulic hand lever to rotate borer.

- Release to stop.

Apply a slight downward pressure onto machine and hold steady.

- Keep drawing out the auger from the hole to clear the excavated earth.

STOPPING AND AFTER USE:

- Turn off engine switch or push stop button.

- Push up collar and release auger from machine.

SAFETY PRECAUTIONS:

- Use only in well-ventilated areas.

- Stop engine when refuelling.

- Securely hold the machine with both hands

HAZARDS:

- Avoid wearing any loose clothing that may tangle in the auger.

- Check for underground power cables, water pipes and tree roots etc before boring holes.

- Take extra care on wet and slippery surfaces.

REQUIRED SAFETY EQUIPMENT