Predator 360 Stump Grinder – Instructions for use

PRE-START CHECKs:

- Position the stump grinder adjacent to the sump before starting the motor.

OPERATION:

- Engage choke

- Set the ON/OFF switch in the “I” or “on” position.

- Set the throttle to start position (min speed) & push the Dead Mans Lever down.

- Pull the start cord. Once the engine has started gradually increase the throttle & disengage the choke.

- When grinding maintain a constant pressure on the stump by regulating the pressure through the handles. Do not let the engine stall.

- Move the grinding head left to right to grind the stump.

- As the stump is ground down, secure the swivel lock, disengage the brake, move the machine forward, reengage the brakes, disengage the swivel lock and continue grinding.

STOPPING AND AFTER USE:

- Reduce the throttle to idle.

- Secure the swivel lock & brakes.

- Release the Dead Mans Handle.

- Turn the start/stop switch to “off”.

- Drop the support leg down into place.

- Wait for the grinding head to stop.

SAFETY PRECAUTIONS:

- Ensure the brake is engaged before disengaging the swivel lock & grinding the stump.

- Never place hands or feet near the grinding disk.

- Always stand behind the machine.

- Do not use on uneven ground.

- Never leave the engine running when unattended.

- Do not operate on a slope more than 11-degree angle.

- The “Dead Mans Handle” must be pushed down when the machine is in operation.

- Never grind from the top of the stump or use the bottom of the grinding head.

REQUIRED SAFETY EQUIPMENT: