Submersible Pump– Instructions for use

PRE-START CHECKs:

- Ensure pump has a current electrical tag.

- Ensure you have the correct type of pump for the job, you need to consider things like how high you need to lift the liquid and how far you need it to travel.

- Treat all pumps & hoses as if you don’t know where they’ve been in the past, they may have been used to pump sewerage or other infectious materials.

- Ensure transformer or a residual current device (RCD) unit is used.

- Ensure electric extension lead is in good condition & positioned safely.

- Check hoses for holes.

- Ensure you comply with your local council authorities regarding where your discharge will end up (liquids always flow downhill. Could this cause a hazard away from your site?)

- If you are not sure please ask.

SETTING UP PUMP:

- Connect discharge hoses to pump (ensure all connections have rubber seal fitted). Lower pump into position by chain or rope fitted to pump (never lower by discharge hoses).

- If you wish to empty your liquid containment fully position the pump at the lowest point but always try & avoid placing in the mud or sludge as this could block up the intake to the pump.

- Run hoses out so the discharge will flow where required.

OPERATION:

- Once the pump has been fully set up plug the power lead into a 240v power supply & turn on.

- You will notice the discharge hoses filling up as the pump forces the liquid through them.

- Follow the discharge hoses from start to finish to ensure they have no sharp bends or kinks in them.

STOPPING AND AFTER USE:

- Turn off power supply.

- Remove pump by lifting out by chain or rope (don’t lift by discharge hose)

- Wash down pump & hoses if required.

- Disconnect hoses & roll up.

- Wind up power lead.

REQUIRED SAFETY EQUIPMENT: