Wallpaper Steamer –  Instructions for use

PRE-START CHECKs:

- Check EC tag is present & current.

- Check electrical cord is undamaged.

- Make sure the unit is switched off before connection to the power.

- Make sure the electrical extension leads are heavy duty & do not exceed 6m in length.

OPERATION:

- Fill water reservoir to “full” mark and place the steam plate face up.

- Connect to power & switch unit on and aloe to heat up. This could take 10+ minutes.

- Once a steady stream of steam is coming from the plate the steamer is ready for use.

- Start at the top of the wall working on an area n more then 1-meter square heating the surface & using a broad blade of scrape to remove paper.

- If the paper is hard to remove, consider holding the steamer over the surface for longer.

STOPPING AND AFTER USE:

- Once completed turn steamer off & allow to cool down. DON’T remove cap from steamer until cooed down as there will be pressure in the tank.

- One cooled down remove cap & empty water tank & remove any wallpaper debris from the unit.

SAFETY PRECAUTIONS:

- Always take care near the steam plate, steam can cause burns.

- Avoid running the unit out of water. When you need to refill the water reservoir turn the unit off & wait

3-5mins for the unit to cool down. Opening the tank can release built up steam.

REQUIRED SAFETY EQUIPMENT: